Guide Specification for

World Dryer Corporation SANITIZE+DRY™

At WORLD DRYER, we’re on a never-ending mission to build solutions upon our customers’ challenges and insights without compromising the pillars of safety, sanitation, performance and compliance. We manufacture a broad portfolio of products designed to fit the needs of any commercial restroom environment, all of which focus on hygiene and offer low cost of ownership and environmental savings. Our goal is to offer the cleanest, safest and most environmentally friendly hand dryers in the world.

This Section specifies SANITIZE+DRY™ by World Dryer Corporation. Features include:

● Sanitizing: Kills 99.99% of common germs in the drying air and on surfaces exposed to the drying air. Laboratory tests confirm 99.99% efficacy against Influenza A, SARS-CoV-2 (COVID-19), Norovirus, Rhinovirus (human cold virus), E. coli, Staphylococcus Aureus, Methicillin-Resistant Staphylococcus Aureus (MRSA), and Enterococcus Faecium. Results project efficacy against a broad range of illness-causing germs; efficacy against a different specific germ may be less. Efficacy is achieved within a few seconds of exposure. As with any effective germicidal feature, actual efficacy is dependent on intensity of exposure and exposure duration. Tests were performed in ISO/IEC 170125 accredited laboratories by personnel qualified in performing microbiological efficacy testing to established test standards.

● Energy Efficient: Optimized technology uses only 3.16 watt-hrs. per use and dries hands in as little as 12 seconds.

● Refined Acoustics: 30% quieter operation with adjustable speed that turns down the volume for less disturbance to adjacent areas.

● Reliable Performance: Motor lasts up to three times longer than the competition.

● Universal Voltage: Smart technology enables hand dryers to adapt to almost any building’s voltage.

● ADA Compliant: Surface mount hand dryer installation complies with ADA, no need for expensive recess kits or additional installation expenses.

Currently unavailable in California and Canada.

For assistance in editing this specification contact World Dryer Email: custservice@worlddryer.com Phone: (800) 323-0701.

This Guide Specification must be carefully edited by a qualified design professional to meet the requirements of the project, ensure compliance with governing building codes, and coordinate with other specification sections and Drawings. Accuracy is the responsibility of the specifier or designer. Retain or delete options, remove brackets, and delete blue guidance notes before publication.

SECTION 102814 – ELECTRIC HAND DRYERS

GENERAL

SUMMARY

Section Includes:

Sanitizing electric hand dryers.

RELATED REQUIREMENTS

Specifier: If retaining this optional Article, revise to match Project Manual section numbers and titles.

        - Division 06 Section "Rough Carpentry" for concealed wood blocking for anchoring electric hand dryers.

Division 09 Section "Non-Structural Metal Framing" for concealed metal backer plates for anchoring electric hand dryers.

Division 26 Section "Grounding and Bonding Requirements for Electrical Systems."

REFERENCES

Specifier: If retaining this optional Article, edit list to correspond to references retained after editing.

References, General: Versions of the following standards current as of the date of issue of the project or as required by applicable code apply to the Work of this Section.

European Standard (EN):

EN 60529 - Degrees of protection provided by enclosures (IP Code).

International Code Council (ICC):

ICC/ANSI A117.1 - American National Standard for Accessible and Useable Buildings and Facilities.

International Organization for Standardization:

ISO/IEC 17025 - Testing and Calibration Laboratories

United States Department of Justice:

ADA - Americans with Disabilities Act, Standards for Accessible Design.

      - ACTION SUBMITTALS

        - Product Data: Include the following:

Operating instructions and performance.

Installation methods.

Electrical wiring diagrams.

INFORMATIONAL SUBMITTALS

Warranty: Sample of Special Manufacturer's Warranty.

Sustainable Design Submittals: Submit verification of compliance with sustainability requirements categories listed in Part 2.

CLOSEOUT SUBMITTALS

Maintenance and cleaning instructions.

QUALITY ASSURANCE

Manufacturer Qualifications: Company specializing in manufacturing of specified products with minimum five years' experience.

DELIVERY, STORAGE, AND HANDLING

Deliver electric hand dryers in manufacturer's original packaging marked for location of installation.

Comply with manufacturer’s instructions for storage and handling.

COORDINATION

Coordinate location of concealed blocking required for anchoring of electric hand dryers.

WARRANTY

Special Manufacturer’s Warranty: Provide manufacturer's standard form in which manufacturer agrees to repair or replace products that fail in materials or workmanship within the following period:

Warranty Period: 10 year limited warranty excluding filters, motor brushes, and accent lighting.

PRODUCTS

MANUFACTURER

Provide list products of World Dryer Corporation, 3602 W. Washington Street, Suite 200, Phoenix, AZ 85009; 800-323-0701; www.worlddryer.com.

Specifier: Retain one of two options in paragraph below.

        - Substitutions: [None acceptable] [In accordance with Instructions to Bidders and Division 01 General Requirements].

PERFORMANCE REQUIREMENTS

Certification: Products certified to Underwriters Laboratory (UL), Inc. and Canadian Standards Association (CSA) national standards for safety; bear UL or ETL markings.

Dryer Efficacy: Kills 99.99% of common viruses and bacteria in the drying air and on surfaces exposed to the drying air including Influenza A, SARS-CoV 2 (Covid-19), Norovirus, Rhinovirus (human cold virus), E Coli, Staphylococcus Aureus, Methicillin-Resistant Staphylococcus Aureus (MRSA) and Enterococcus Faecium as a tested by an ISO/IEC 170125 accredited laboratory.

        - Accessibility Requirements: Comply with requirements of ADA/ABA and accessibility requirements of authorities having jurisdiction.

Comply with ICC/ANSI A117.1.

SUSTAINABILITY REQUIREMENTS

Specifier: Edit Sustainability Requirements to correspond to sustainability certification program and categories that are applicable to Project.

Sustainability Requirements: Comply with the requirements for material selection categories applicable to Project including the following:

Minimum Energy Performance.

Optimize Energy Performance.

Building Product Disclosure and Optimization - Environmental Product Declaration.

ELECTRIC HAND DRIERS

Sanitizing Hand Dryer: High-speed energy-efficient rapid-drying heated electric hand dryer, with automatic sensor, adjustable speed and sound control.

Basis of Design Product: Sanitize+Dry™ by World Dryer Corporation.

Mounting: Surface mounted.

Cover Material:

Specifier: Retain one of two subparagraphs below. Retain one of two options in first subparagraph below if retained.

One piece, heavy duty, die-cast aluminum, [white enamel] [black enamel] finish.

One piece, vandal resistant, stainless steel, brushed finish.

Electrical rating: 110-120/208/220-240 VAC input, 950 watts; 277 VAC, 1150 watts.

Nozzle type: High intensity single port.

Air flow: 64 CFM at 225 MPH (19,800 LFM).

Consumables: No chemicals, HEPA filers, or UV lamps required.

Intake air filter: Standard (non-HEPA) type, replaceable.

Motor: Thermally protected universal brush type, 31,000 RPM, with replaceable brushes.

Controls:

Activation by automatic infrared sensor with 120 second vandal shutoff.

High/Medium/Low speed control switch.

Heater:

Nichrome wire element type with automatic resetting thermal cutoff.

Heated air temperature: 130 degrees F at 70 degrees F ambient temperature.

Sound Level (Adjustable): 69 dBA at 79 in. (2 m).

Ingress Protection Rating: EN 60529: IP24.

ACCESSORIES

Recess Mounting Kit: Not required for ADA/ABA compliance.

- EXECUTION

      - EXAMINATION

        - Examine substrates and conditions for suitability for installation of electric hand dryers. Proceed with installation once conditions are in accordance with manufacturer's written instructions.

      - INSTALLATION

        - Install electric hand dryers in locations and heights indicated on Drawings, in accordance with manufacturer's written instructions, using manufacturer's recommended fasteners. Install units level, plumb, and firmly anchored in locations and at heights indicated. Connect to electrical power.

CLEANING AND TESTING

Remove protective packaging.

Clean surfaces with mild soap solution. Do not use abrasives.

Inspect installed dryers to verify mounting is rigid and electrical connections are proper.

Test each dryer to verify operation and performance.

PROTECTION

Protect dryers from construction activities until just prior to Substantial Completion.

END OF SECTION